SERVER ROOM

MONITORING SYSTEM

Product Requirements Document (PRD)

Version 1.0

Date: 24/6/2026

# 1. PROJECT OVERVIEW

Project Name: Server Room Monitoring System

Objective: Real-time monitoring of server room environmental conditions including temperature, humidity, and gas levels with centralized dashboard and alerting system

Target Users: Server administrators, IT operations, facility managers

# 2. PROBLEM STATEMENT

- Server rooms require constant environmental monitoring to prevent hardware failures and safety risks

- Manual monitoring is inefficient and error-prone

- Early detection of temperature/humidity anomalies prevents costly hardware damage

- Gas leak detection (CO, LPG) is critical for personnel safety

# 3. SOLUTION OVERVIEW

An integrated IoT monitoring system combining edge sensors (ESP32), backend server (Raspberry Pi 4), REST API, and web-based dashboard for real-time environmental monitoring with historical data analysis and alerting capabilities.

## 3.1 System Architecture

The system consists of three main layers:

| Layer | Description |
| --- | --- |
| Edge Sensors | ESP32 + DHT22, MQ-9 sensors |
| Backend Server | Raspberry Pi 4 with Node.js API & PostgreSQL |
| Frontend | React-based web dashboard |

# 4. HARDWARE COMPONENTS

## 4.1 Edge Device (ESP32)

| Component | Specification |
| --- | --- |
| Microcontroller | ESP32 DevKit v1 - Dual core, 240 MHz, WiFi + BLE |
| Storage | 4 MB Flash (SPIFFS) |
| Power | Micro USB 5V input (can use VIN for 5V stability) |
| WiFi | 2.4 GHz 802.11 b/g/n |

Function: Data collection from sensors and transmission to backend server

## 4.2 Sensors

| Sensor | Type | Range | Interface |
| --- | --- | --- | --- |
| DHT22 | Temp + Humidity | -40 to 80°C, 0-100% | GPIO (1-wire) |
| MQ-9 | CO + CH4 | CO: 10-1000 ppm | Analog (ADC) |

## 4.3 Backend Server (Raspberry Pi 4 Type B)

| Component | Specification |
| --- | --- |
| Model | Raspberry Pi 4 Model B (4 GB RAM minimum) |
| CPU | ARM Cortex-A72 Quad-core 1.5 GHz |
| OS | Raspberry Pi OS (Debian-based) |
| Database | PostgreSQL 13+ |
| Runtime | Node.js 18 LTS |
| Storage | 32GB microSD card minimum |

# 5. FUNCTIONAL REQUIREMENTS

## 5.1 Data Collection & Transmission

- ESP32 reads sensor data every 10 seconds

- Data includes: Temperature, Humidity, CO level, LPG level, Timestamp

- Transmission: HTTP POST to Raspberry Pi API every 30 seconds (batched)

## 5.2 Data Storage

- Database: PostgreSQL on Raspberry Pi

- Retention: 1 year of data with 1-minute aggregation

- Backup: Daily automated backups

## 5.3 Alerting System

- Temperature alert: >28°C (WARNING) or >35°C (CRITICAL)

- Humidity alert: <30% or >80%

- CO alert: >35 ppm (WARNING) or >100 ppm (CRITICAL)

- LPG alert: >1000 ppm (WARNING) or >5000 ppm (CRITICAL)

- Notification: Email, Dashboard notification, Buzzer activation

## 5.4 Dashboard Features

- Real-time display of current sensor readings

- Historical charts (24h, 7d, 30d, 1y views)

- Alert history and management

- Min/Max/Average statistics

- System health status

- Export functionality (PDF, CSV)

# 6. API DESIGN

## 6.1 Base URL

http://raspberrypi.local:3000/api/v1

## 6.2 Core Endpoints

| Method | Endpoint | Description |
| --- | --- | --- |
| POST | /sensor-data | Submit sensor readings from ESP32 |
| GET | /sensor-data/latest | Get most recent sensor data |
| GET | /sensor-data/history | Get historical data with date range |
| GET | /alerts | Fetch triggered alerts |
| GET | /system/status | Get system health status |

## 6.3 POST /sensor-data (Sensor Data Submission)

Request Body:

{ "temperature": 25.5, "humidity": 55, "co_ppm": 12, "lpg_ppm": 200, "timestamp": "2025-01-15T10:30:00Z" }

Response: { "success": true, "message": "Data stored successfully" }

## 6.4 GET /sensor-data/latest

Returns the most recent sensor reading from the database

## 6.5 GET /sensor-data/history

Query params: ?start_date=2025-01-01&end_date=2025-01-15&limit=100

Returns historical data for charting and analysis

# 7. DATABASE DESIGN

## 7.1 Technology: PostgreSQL

Chosen for reliability, performance, and JSONB support for flexible data storage

## 7.2 Primary Tables

| Table | Key Columns | Purpose |
| --- | --- | --- |
| sensor_readings | id, temperature, humidity, co_ppm, lpg_ppm, recorded_at | Store raw sensor data |
| alerts | id, type, severity, value, created_at, resolved | Track triggered alerts |
| alert_config | id, alert_type, warn_threshold, crit_threshold | Store alert thresholds |
| users | id, email, password_hash, role | User accounts |

## 7.3 Sample Queries

Get average temperature for last 24 hours:

SELECT AVG(temperature) FROM sensor_readings WHERE recorded_at > NOW() - INTERVAL '24 hours'

Fetch all critical alerts:

SELECT * FROM alerts WHERE severity='CRITICAL' ORDER BY created_at DESC

# 8. TECHNOLOGY STACK

| Layer | Technology | Purpose |
| --- | --- | --- |
| Firmware | Arduino (C++) | ESP32 sensor code |
| Backend | Node.js + Express | REST API server |
| Database | PostgreSQL | Data persistence |
| Frontend | React + TypeScript | Web dashboard |
| Charts | Recharts | Data visualization |
| Auth | JWT | API authentication |

# 9. DASHBOARD FEATURES

## 9.1 Homepage

- Real-time gauge charts for Temperature, Humidity, CO, LPG

- Status indicators: GREEN (normal), YELLOW (warning), RED (critical)

- Last update timestamp

- System connectivity status (ESP32, API)

## 9.2 Analytics Page

- Line charts showing 24h, 7d, 30d trends

- Min/Max/Average statistics for selected period

- Downloadable data as CSV/JSON

- Custom date range selection

## 9.3 Alerts Page

- List of triggered alerts with timestamp, type, and severity

- Filter by alert type and date range

- Mark alerts as read/resolved

- Export alert logs

## 9.4 Settings Page

- Configure alert thresholds

- Email notification settings

- API key management

- User account management

# 10. NON-FUNCTIONAL REQUIREMENTS

| Requirement | Specification |
| --- | --- |
| Performance | API response time <200ms, Dashboard load <2s |
| Availability | 99.5% uptime, automated failure recovery |
| Security | HTTPS/TLS, JWT authentication, input validation |
| Scalability | Support 10+ ESP32 nodes simultaneously |
| Data Retention | 1 year of data with automatic backup |
| Reliability | Zero data loss, automatic database repair |

# 11. IMPLEMENTATION TIMELINE

| Phase | Duration | Deliverables |
| --- | --- | --- |
| Hardware Setup | 1 week | ESP32 + sensors assembled and tested |
| Firmware Development | 2 weeks | Sensor code + WiFi communication |
| Backend API | 3 weeks | Node.js API + PostgreSQL database |
| Frontend Dashboard | 3 weeks | React UI + charts + alerts |
| Testing & Deployment | 2 weeks | UAT + production deployment |

# 12. RISK ASSESSMENT & MITIGATION

| Risk | Severity | Mitigation |
| --- | --- | --- |
| Network disconnection | High | Implement local data buffering, auto-reconnect logic |
| Sensor calibration drift | Medium | Quarterly calibration checks, trending alerts |
| Database corruption | High | Daily automated backups, transaction logging |
| Raspberry Pi failure | High | UPS backup, spare unit available |
| API rate limiting | Low | Implement request throttling, caching |

# 13. DEPENDENCIES

- Network connectivity (WiFi/Ethernet on Raspberry Pi)

- Stable power supply (UPS recommended)

- Proper sensor calibration

- Adequate ventilation around sensors

# 14. SUCCESS CRITERIA

- System uptime: 99.5% availability

- Data accuracy: ±0.5°C for temperature, ±3% for humidity

- Alert notification latency: <5 minutes

- Dashboard load time: <2 seconds

- Zero data loss over 1-year operation

# APPENDIX: PIN CONFIGURATION

## ESP32 Pin Wiring

| Component | Pin | Function |
| --- | --- | --- |
| DHT22 VCC | 3.3V | Power |
| DHT22 DATA | GPIO 4 | Temperature + Humidity |
| DHT22 GND | GND | Ground |
| MQ-9 AOUT | GPIO 34 (via 20k+10k divider) | CO analog reading |
| MQ-9 DOUT | GPIO 35 | CO digital threshold |
| MQ-9 VCC | VIN (5V) | Power |
| MQ-9 GND | GND | Ground |
| Buzzer + | Via NPN Transistor | Alarm output |
| Transistor Base | GPIO 18 | Buzzer control |
| LCD SDA | GPIO 21 | I2C data |
| LCD SCL | GPIO 22 | I2C clock |